# FDTD Equations

Maxwell’s equations can be expressed compactly in terms of the curl operator:

∇×E=-μ0∂∂tH ∇×H=ϵ∂∂tE

For a non-dispersive (lossless), isotropic, non-magnetic material in 3D, we can write this explicitly in terms of the (x,y,z) vector components:

∂∂tEx=1ϵ∂∂yHz-∂∂zHy ∂∂tHx=-1μ0∂∂yEz-∂∂zEy∂∂tEy=1ϵ∂∂zHx-∂∂xHz ∂∂tHy=-1μ0∂∂zEx-∂∂xEz∂∂tEz=1ϵ∂∂xHy-∂∂yHx ∂∂tHz=-1μ0∂∂xEy-∂∂yEx

On classical computers employing the FDTD method, the E and H time derivatives are discretized with the explicit forward-Euler formula.  The E and H grids are offset by one-half grid length in x, y, and z to make a convenient central finite difference approximation of the spatial derivatives.

In FDTD, time updates for E and H variables are staggered by a half time step: first E is updated, then H, then E again, and so on.  We don’t need this for quantum computation since we will be simulating the Hamiltonian directly from the state transition matrix. We now proceed to derive the entries of this matrix.

## 2D Case

Consider a cavity with PEC planes on the top and bottom and perfect magnetic boundaries on the edges.  Let the z-dimension be much less than its x,y extents.

PEC

PMC

PMC

x

y

z

Figure 1.  A cavity model with PEC on the upper and lower z-boundaries and PMC on the (x, y) boundaries.

Then at a low enough frequency, only Hx, Hy, and Ez are non-zero and they are independent of z.  Maxwell’s equations now reduce to:

∂∂tEz=1ϵ∂∂xHy-∂∂yHx ∂∂tHx=-1μ0∂∂yEz  ∂∂tHy=1μ0∂∂xEz

We impose a regular grid of size nx×ny with points xi,yj=(ih,jh), where h is the grid spacing and i∈[0, nx-1], j∈[0, ny-1], and let Ez take on the values Ez(i,j) at these points.  Thus, the size of the domain is hnx-1×h(ny-1).

The magnetic fields are discretized on offset grids.  The field Hx is defined at grid points xi,yj=(ih,jh+h2) for i∈[0, nx-1], j∈[0, ny-2].  The field Hy is discretized on the offset grid (xi, yj)=(ih+h2,jh) for i∈[0, nx-2], j∈[0, ny-1].

Hy(i,j)

Hy(i-1,j)

Ez(i,j)

Ez(i+1,j)

Ez(i-1,j)

Hx(i,j)

Hx(i,j-1)

Ez(i,j+1)

Figure 2. Finite difference discretization scheme, showing offset grids for Ez (dark circles) and Hx and Hy (light circles).

If we now discretize the 2D Maxwell equations with a central difference formula, we obtain:

∂∂tEz(i,j)=1hϵHyi,j-Hyi-1,j-Hxi,j+Hxi,j-1
∂∂tHx(i, j)=-1hμ0Ezi,j+1-Ez(i,j)
 ∂∂tHy(i,j)=1hμ0Ezi+1,j-Ez(i,j)

At a PMC boundary, Hx or Hy goes to zero, and no degrees of freedom are created.  Taking for example the case of the x=0 boundary where the DoF Hy(i-1,j) does not exist, we introduce the approximation Hyi-1,j=-Hy(i,j). This implies that the average value of the field at the boundary is [Hy (i-1)+Hy (i,j)]/2=0, which is precisely what we want for the PMC.  Then the equation for Ez at this boundary is accordingly modified to

∂∂tEz(i,j)=1hϵ2Hyi,j-Hxi,j+Hxi,j-1

The other PMC boundaries are handled in an analogous way.

The discretized equations have the following matrix forms:

∂∂txE=AExH

∂∂txH=AHxE

Here xE contains the {Ez} variables and xH contains the {Hx, Hy} variables.  Matrix AE has dimensions N×M where N=nxny and M=nxny-1+nx-1ny.  The matrix AH has dimensions M×N.

An equivalent block form of the equations is

∂∂txExH=0AEAH0xExH

Therefore, given some initial states xE0 and xH(0), the solution at time t can be computed using the matrix exponential:

xE(t)xH(t)=exp0AEAH0txE(0)xH(0)